3.1	REQUIREMENTS ANALYSIS

The Requirements Analysis phase translates the operational challenges and user needs identified in the problem analysis into a technical framework for CrayCare. This section defines the system's functional and non-functional requirements through the Requirements and Features Matrix, Use Case Diagram, and Use Case Reports. It connects the needs of small-scale crayfish farmers with the monitoring, production-recording, feeding, control, notification, analytics, and account-management capabilities implemented in the system.

3.1.1	Requirements and Features Matrix

The Requirements and Features Matrix maps each functional and non-functional requirement to the corresponding CrayCare feature. It also identifies the priority and implementation status of each requirement.

Table 3.1

Functional Requirements Matrix

| Hardware Requirements |  |  |  |
| --- | --- | --- | --- |
| Requirement Description | System Feature(s) | Priority | Status |
| The system must monitor water temperature in real time. | Real-time water-temperature monitoring using a DS18B20 waterproof temperature sensor. | High | Implemented |
| The system must monitor water pH in real time. | Real-time pH monitoring using a DFRobot Gravity analog pH sensor. | High | Implemented |
| The system must monitor water turbidity in real time. | Real-time turbidity monitoring using a DFRobot Gravity analog turbidity sensor module. | High | Implemented |
| The system must monitor dissolved oxygen (DO) levels in the water in real time. | Real-time dissolved oxygen monitoring in the water using a DFRobot Gravity analog dissolved oxygen sensor. | High | Implemented |
| The system must monitor tank water level in real time. | Real-time tank water-level monitoring using an ultrasonic sensor. | High | Implemented |
| The system must monitor the feeder hopper level in real time. | Real-time feeder hopper-level monitoring using an ultrasonic sensor. | High | Implemented |
| The system must transmit sensor data over Wi-Fi in real time. | Periodic Wi-Fi transmission of the latest sensor readings to the application every 10 seconds. | High | Implemented |
| The system must control the water pump to circulate tank water through the connected filtration setup. | Relay-controlled water circulation through the connected filtration setup. | High | Implemented |
| The system must aerate crayfish tanks. | Relay-controlled tank aeration. | High | Implemented |
| The system must automate feeding in the grow-out tank. | Servo-operated automatic feed dispensing. | High | Implemented |
| The system must automatically switch the aerator and water pump between ON and OFF states in AUTO mode based on configured thresholds and valid sensor readings. | Threshold-based automatic ON/OFF control of the aerator and water pump using valid sensor readings. | High | Implemented |

| Software Requirements |  |  |  |
| --- | --- | --- | --- |
| Requirement Description | System Feature(s) | Priority | Status |
| The system must allow secure user authentication and account recovery. | Email/password and Google authentication with email verification, password reset, and account-status checks. | High | Implemented |
| The system must support role-based user access. | Role-based access for the System Admin and Tank Owners. | High | Implemented |
| The system must restrict tank-operation write actions to the designated owner. | Owner-only write access to tank monitoring and control records. | High | Implemented |
| The system must allow the administrator to manage owner accounts and hardware assignment. | Owner-account management and hardware assignment for one active tank. | High | Implemented |
| The system must allow the assigned tank owner to view current sensor readings and device connectivity remotely. | Remote sensor monitoring with device and data-connectivity indicators. | High | Implemented |
| The system must record the initial tank and baseline sampling setup. | Tank initialization with stocking and baseline sampling records. | High | Implemented |
| The system must allow the assigned tank owner to configure water-quality, water-level, and feed-level thresholds. | Configurable water-quality, water-level, and feed-level thresholds with input validation. | High | Implemented |
| The system must record sampling data. | Stores individual crayfish weight and length measurements; the app computes sample size, totals, averages, and biomass for display and reports. | High | Implemented |
| The system must perform Water Quality Anomaly Detection (WQAD) on multivariate water-sensor patterns and explain unusual results. | Every 10 minutes, the Isolation Forest analyzes valid temperature, pH, dissolved oxygen, and turbidity readings in a rolling two-hour window, requiring at least six readings spanning 110 minutes and tolerating one missed interval. Results include anomaly status, percentile score, ranked contributors, insight, and recommendation. | High | Implemented |
| The system must show crayfish growth and production trends. | Growth and production views showing sample length, sample weight, biomass, survival rate, mortality, and culture-day summaries. | Medium | Implemented |
| The system must allow the assigned tank owner to manage recurring feeding schedules and view advisory water-quality warnings in the Feeding controls. | Owners can add, edit, delete, enable, or disable recurring schedules. The Feeding controls show water-quality warnings for stale, unavailable, invalid, or out-of-range readings. | Medium | Implemented |
| The system must allow the assigned tank owner to dispense Feed Now in whole-number amounts from 1 to 200 g, subject to feeder, feed-level, and schedule safety checks; water-quality conditions produce warnings only. | Feed Now validates whole-gram amounts, feeder and connection readiness, schedule timing, and fresh, valid feed-level data above the critical threshold. Stale, unavailable, invalid, or out-of-range water-quality readings produce advisory warnings. | Medium | Implemented |
| The system must track daily feeding progress. | Daily feeding progress with completed, previous, and upcoming feeding details. | Low | Implemented |
| The system must generate downloadable PDF and Excel reports for selected batches, including batch summaries and optional sampling, individual crayfish measurements, mortality, and harvest details. | PDF and Excel export for selected batches, with selectable batch summaries, sampling records, individual weight-and-length measurements, mortality records, and harvest records. | Medium | Implemented |
| The system must notify users of abnormal water conditions, operational status changes, feeding reminders, and sampling reminders. | Push and in-app alerts for water conditions, operations, feeding, and sampling. | Medium | Implemented |
| The system must store historical sensor data. | Cloud-based storage of historical sensor readings. | Medium | Implemented |
| The system must display the latest available sensor data during network interruptions. | Offline access to cached sensor readings. | Medium | Implemented |
| The system must allow the assigned tank owner to set each actuator to OFF, AUTO, or ON. | OFF, AUTO, and ON actuator modes with offline protection and activity logs. | Medium | Implemented |
| The system must provide historical water-sensor analytics. | Historical sensor charts with minimum, maximum, and average values across selectable time ranges. | Medium | Implemented |
| The system must allow users to manage their profiles and credentials. | Profile and password management based on the user's sign-in method. | Medium | Implemented |
| The system must manage notifications and owner preferences. | Notification history, read status, category filters, and alert preferences. | Medium | Implemented |
| The system must manage production and batch records. | Batch, sampling, mortality, harvest, population, and completion records. | High | Implemented |
| The system must maintain feeding and actuator activity logs. | Feeding and actuator logs with timestamps, outcomes, and failure or skip details. | Medium | Implemented |
| The system must continue synchronized feeding schedules and automatic actuator control during temporary internet interruptions. | Local execution by the ESP32 of synchronized feeding schedules and AUTO actuator rules during temporary internet interruptions. | High | Implemented |

Table 3.1 presents the functional requirements and corresponding CrayCare features. These include sensor monitoring and threshold configuration; Wi-Fi data transmission; pump circulation and aeration; scheduled and manual feeding with advisory water-quality warnings and feed-level safety checks; user authentication and role-based access; tank, sampling, mortality, harvest, and growth management; Water Quality Anomaly Detection (WQAD) with an Isolation Forest trained on an external freshwater-pond dataset; notifications; historical analytics; and PDF and Excel reports with optional individual crayfish measurements.

Table 3.2

Non-Functional Requirements Matrix

| Non-Functional Requirements |  |  |  |
| --- | --- | --- | --- |
| Requirement Description | System Feature(s) | Priority | Status |
| Hardware must operate reliably under the tank’s expected moisture, splash, and installation conditions. | ESP32, sensors, enclosure, and tank installation. | Medium | Implemented |
| Valid ten-minute sensor-history records must be retained when the ESP32 temporarily loses Wi-Fi and synchronized after reconnection. | The ESP32 buffers ten-minute history records locally in SD/LittleFS and synchronizes them after reconnection. | High | Implemented |
| Sensor and production records must reject invalid data and avoid duplicate or inconsistent outcomes during retries. | Firestore validation, transactions for relevant updates, and duplicate checks for buffered history uploads. | High | Implemented |
| Under stable connectivity, new sensor readings should appear on the dashboard within five seconds of the Firestore write. | The ESP32 publishes latest readings every 10 seconds; the app uses Firestore listeners and cached readings to refresh the dashboard. | High | Implemented |
| Under stable connectivity, valid actuator and Feed Now commands should be acknowledged within three seconds, with status updates shown within five seconds. | The ESP32 polls Feed Now commands about every second and actuator settings about every five seconds. Feeder status syncs periodically every 15 seconds, with immediate updates on state changes. | High | Implemented |
| Tank owners should be able to review readings, change thresholds, feed, record sampling, and export reports using clear controls and validation feedback. | Guided app screens, labeled controls, and input validation. | High | Implemented |
| PDF and Excel reports must present selected records with descriptive headings, dates, units, and readable values without clipped columns. | Report builders provide section titles, column labels, units, and calculated values across PDF and Excel formats. | Medium | Implemented |
| The system must prevent unauthorized users from accessing or changing farm data outside their authorized role and tank. | Firebase Authentication, role-based access, and owner-scoped Firestore rules for protected data and actions. | High | Implemented |
| The system should maintain compatible and authenticated data exchange among the mobile app, ESP32 hardware, Cloud Firestore, and Cloud Functions. | The Flutter app exchanges data with Firestore and invokes authenticated Cloud Functions. The ESP32 writes live readings and normal history directly to Firestore; buffered offline history is sent through the Cloud Function staging and assignment-validation route. | High | Implemented |

Table 3.2 presents CrayCare’s quality and operating requirements. These include calibrated sensor accuracy, hardware reliability, buffering of ten-minute sensor-history records during Wi-Fi interruptions, dashboard refresh and command-response targets, usability, security, data integrity, maintainability, report readability, and authenticated data exchange through Firebase services.